# MATERIAL REPOSITORY

This section provides users with an overview of how the Hyperelastic Material Repository operates, detailing the structure of the input files in .yaml format and the adopted material classification. The repository is designed to gather, in an organized and standardized way, experimental data from the literature or provided by the user, enabling its direct use in hyperelastic model selection and parameter calibration. This structure allows the program to load experimental datasets dynamically and enables users to expand the repository without modifying the source code. When a .yaml data file is added to the appropriate directory, HyperSmart automatically recognizes it and makes it available for selection in the program. The adopted categorization facilitates the location and comparison of materials, covering different classes and subclasses, in order to meet the needs of research in mechanical, civil, biomedical engineering, and related fields.

## Input File Formatting

The Hyperelastic Material Repository uses input files in the YAML (.yaml) format to store experimental data in a structured and human-readable way. Each file corresponds to a single material dataset and contains both metadata (information about the material, source, and testing conditions) and the experimental data itself.

The main keys and their purposes are:

- material_class – Main classification of the material. Must be one of: "Elastomers", "Foams", "Soft Biological Tissues", or "Gels & Hydrogels".

- material_subclass – Subcategory within the selected material class (e.g., "Animal", "Human", "Polymeric Open-Cell Foams").

- material – Full name of the tested material.

- publication_title, author, year – Bibliographic information of the study from which the data was obtained.

- description – Brief summary of the type of test performed and relevant conditions.

- stress_measure – Stress type used in the dataset. Must be "Nominal" or "Cauchy".

- deformation_modes – A dictionary indicating whether each pure deformation mode is available (true or false for "axial", "biaxial", "simple_shear", and "pure_shear").

- unit_of_measure – Unit of stress measurement for each mode (false if data is not available for that mode).

- data_source – Indicates how the experimental data was obtained:

  - 1 – Data provided directly in the scientific article (tables or supplementary files).

  - 2 – Data extracted from figures using an image extraction tool.

- citation, doi – Full bibliographic citation and DOI link to the source.

- note – Additional remarks regarding the dataset (e.g., special test conditions, assumptions, limitations, or quality considerations).

- data – Nested dictionary containing numerical values for each available deformation mode. Each mode contains:

  - stretch – Array of stretch (λ) or shear parameter values.

  - stress – Corresponding stress values in the defined unit.

Below is shown a qualitative example of the formatting using part of a Dataset for Porcine Liver:

## Material Categorization

To facilitate navigation and ensure that datasets can be quickly located and compared, all materials in the Hyperelastic Material Repository are organized into a hierarchical categorization system. At the top level, materials are grouped into four broad classes — Elastomers, Foams, Soft Biological Tissues, and Gels & Hydrogels. Within each class, materials are further subdivided into subclasses according to their composition, structure, or origin and examples are presented.

This categorization not only helps users filter and identify relevant datasets but also supports consistency when adding new entries to the repository. By following a standardized classification, researchers can efficiently compare materials with similar characteristics, regardless of their source or testing method.

The following sections describe each material class and its corresponding subclasses in detail.

### Elastomers

- Natural Rubber (NR)

- Synthetic Rubbers

  - Hydrocarbon-Based: SBR, BR, EPDM, IIR.

  - Polar Rubbers: NBR, CR.

  - Specialty Synthetics: FKM, ACM.

- Silicone Rubbers (Q)

- Polyurethanes (PU)

  - Thermoset PU.

- Thermoplastic Elastomers (TPE)

  - TPO, TPV, SBS, SEBS, and Thermoplastic PU (TPU).

- Others/Hybrid/Specialty Elastomers

  - TPSiV, Bio-Based Rubbers.

### Foams

- Polymeric Open-Cell Foams

- Polymeric Closed-Cell Foams

- Others

### Soft Biological Tissues

- Human

- Animal

- Others

### Gels & Hydrogels

- Natural Polymer-Based

  - Collagen gels

  - Fibrin gels

  - Alginate hydrogels

  - Hyaluronic acid (HA) hydrogels

- Synthetic Polymer-Based

  - PEG hydrogels

  - PAAm (Polyacrylamide) gels

  - PVA (Polyvinyl alcohol) hydrogels

- Composite/Hybrid

  - Collagen-PEG hybrids

  - Nanocellulose-reinforced hydrogels

- Others

# Hyperelastic models

Hyperelastic models are mathematical formulations used to describe the mechanical behavior of materials capable of undergoing large elastic deformations, such as elastomers, soft biological tissues, foams, and gels. These models differ in their theoretical foundations, functional forms, and range of applicability, but all share the same goal: to provide accurate stress–strain predictions for different deformation modes.

This section is divided into two main parts. The first (Section 2.1) presents the analytical expressions for the most important pure deformation modes across the main categories of hyperelastic models. The second (Section 2.2) provides an extensive literature review, cataloging a wide range of models along with their equations, sources, and classification.

## Pure Deformation Modes Expressions

A fundamental step in working with hyperelastic models is understanding how they express stress–strain relationships under the principal pure deformation modes:

- Uniaxial (axial tension or compression)

- Equibiaxial (equal stretches in two directions)

- Simple shear

- Pure shear

This subsection presents the analytical expressions for each of these deformation modes for the three major types of hyperelastic models:

- Invariant-Based Models (stress derived from invariants)

- Stretch-Based Models (stress expressed directly as a function of principal stretches)

- Hookean-Type Models (generalized Hookean formulations extended to large strains)

The aim is to provide a quick reference for the derivation and application of these expressions, serving as a link between theoretical model definitions and their practical use in material parameter calibration. These expressions are also essential for users who wish to expand the program’s hyperelastic model library, enabling the correct implementation of both existing models from the literature and custom-developed formulations.

### Invariant-Based Models

#### Uniaxial

σ11C=2∂W∂I1+∂W∂I21λλ2-1λ

P11=2∂W∂I1+∂W∂I21λλ-1λ2

#### Biaxial

σ11C=2∂W∂I1+∂W∂I2λ2λ2-1λ4

P11=2∂W∂I1+∂W∂I2λ2λ-1λ5

#### Pure Shear

σ11C=2∂W∂I1+∂W∂I2λ2-1λ2

P11=2∂W∂I1+∂W∂I2λ-1λ3

#### Simple Shear

σ12C=P12=2∂W∂I1+∂W∂I2γ

### Stretch-Based Models

#### Uniaxial

σ11C=λ∂W∂λ1-∂W∂λ31λλ

P11=∂W∂λ1-∂W∂λ31λ3

#### Biaxial

σ11C=λ∂W∂λ1-∂W∂λ31λ2

P11=∂W∂λ1-∂W∂λ31λ3

#### Pure Shear

σ11C=λ∂W∂λ1-∂W∂λ31λ

P11=∂W∂λ1-∂W∂λ31λ2

#### Simple Shear

σ12C=P12=∂W∂λ12+γ2γ2+4+γ-∂W∂λ32-γ2γ2+4-γ4+γ2

### Hookean-type Models

#### Uniaxial

P11=z fλ1 f'λ1→ z=3.L1.L2+2.L22L1+L2

σ11C=zfλ1f'λ1λ22

ν=-ε22ε11=-ε33ε11

#### Biaxial

P11=z fλ1 f'λ1→ z=3L1L2+2L22L1+L2

σ11C=P11λ1λ3

ν=ε33-2ε11+ε33=ε33-2ε22+ε33

#### Pure Shear

P11=z fλ1 f'λ1→ z=4L1L2+4L22L1+2L2

σ11C=P11λ3

ν=ε33-ε11+ε33

#### Simple Shear

P12=σ12C=f'λ12λ1cosβ+γ1+sinβ2L2fλ1+L1fλ1+fλ2-

-f'λ22λ2cosβ+γsinβ-12L2fλ2+L1fλ1+fλ2

λ1=1+sinβcosβ;     λ2=1-sinβcosβ

## Literature Review of Hyperelastic Models

This subsection compiles a comprehensive set of 106 hyperelastic models from the scientific literature, organized according to their theoretical basis and functional form. For each model, the equation, name, publication source and material parameters.

The models are grouped into three main categories and corresponding subcategories:

1 - Phenomenological Models

Principal Stretch-Based Models

Invariant-Based Models

Mixed Phenomenological Model

Series Function Models

Limiting Chain Extensibility Models

Power Law, Exponential or Logarithmic Function Models

2 - Micromechanical Network Models

Gaussian Chain Network Models

Non-Gaussian Chain Network Models

Mixed Micromechanical Network Models

3 - Hookean-type Models

This structured classification enables users to identify models relevant to their research focus, compare their mathematical complexity, and trace their origins in literature. It also supports the addition of new models to the repository by following consistent formatting and categorization.

### Phenomenological Models

Phenomenological models describe the macroscopic mechanical response of hyperelastic materials, as tested experimentally, using mathematical functions chosen for their ability to fit experimental data, without directly representing the material’s microstructure. They are widely used due to their flexibility, simplicity, and broad applicability across different material types.

#### Stretch-Based Models

| Model Name: Valanis-Landel | Source: (Valanis; Landel, 1967) |
| --- | --- |
| Equation: W=2μi=13 λiln⁡λi-1 |  |
| Parameters: μ |  |

| Model Name: Peng-Landel | Source: (Peng; Landel, 1972) |
| --- | --- |
| Equation: W=E∑i=13 λi-1-ln⁡λi-16ln⁡λi2+118ln⁡λi3-1216ln⁡λi4 |  |
| Parameters: E |  |

| Model Name: Ogden | Source: (Ogden, 1972) |
| --- | --- |
| Equation: W=∑i=1N 2μiαi2λ1αi+λ2αi+λ3αi-3 |  |
| Parameters: αi,μi |  |

| Model Name: Attard | Source: (Attard; Hunt, 2004) |
| --- | --- |
| Equation: W=∑i=1N Ai2iλ12i+λ22i+λ32i-3+Bi2iλ1-2i+λ2-2i+λ3-2i-3,N=1,2,3… |  |
| Parameters: Ai,Bi |  |

| Model Name: Shariff | Source: (Shariff, 2000) |
| --- | --- |
| Equation: W=∑i=13 ωλi,ωλi=E∑j=0n αjφjλi |  |
| Parameters: E,α1,α2,α3,α4 |  |

| Model Name: Arman-Narooei | Source: (Narooei; Arman, 2018) |
| --- | --- |
| Equation: W=∑i=1N Aiemiλ1αi+λ2αi+λ3αi-3-1+∑j=1N Bjenjλ1-βj+λ2-βj+λ3-βj-3-1,N=1,2,3… |  |
| Parameters: Ai,Bj,mi,nj,αi,βj |  |

#### Invariant-Based Models

| Model Name: Mooney-Rivlin | Source: (Mooney, 1940) |
| --- | --- |
| Equation: W=C10I1-3+C01I2-3 |  |
| Parameters: C10,C01 |  |

| Model Name: Neo-Hookean | Source: (Treloar, 1943b) |
| --- | --- |
| Equation: W=C10I1-3 |  |
| Parameters: C10 |  |

| Model Name: Isihara | Source: (Isihara; Hashitsume; Tatibana, 1951) |
| --- | --- |
| Equation: W=C10I1-3+C20I1-32C01I2-3 |  |
| Parameters: C10,C20,C01 |  |

| Model Name: Biderman | Source: (Biderman, 1958) |
| --- | --- |
| Equation: W=C10I1-3+C20I1-32C01I2-3 |  |
| Parameters: C10,C01,C20,C30 |  |

| Model Name: James-Green-Simpson | Source: (James; Green; Simpson, 1975) |
| --- | --- |
| Equation: W=C10I1-3+C01I2-3+C11I1-3I2-3+C20I1-32+C30I1-33 |  |
| Parameters: C10,C01,C11,C20,C30 |  |

| Model Name: James-Green-Simpson | Source: (Haines; Wilson, 1979) |
| --- | --- |
| Equation: W=C10I1-3+C01I2-3+C11I1-3I2-3+C02I2-32+C20I1-32+C30I1-33 |  |
| Parameters: C10,C01,C11,C02,C20,C30 |  |

| Model Name: Yeoh | Source: (Yeoh, 1990) |
| --- | --- |
| Equation: W=∑i=13 Ci0I1-3i |  |
| Parameters: C10,C20,C30 |  |

| Model Name: Lion | Source: (Lion, 1997) |
| --- | --- |
| Equation: W=C10I1-3+C01I2-3+C50I1-35 |  |
| Parameters: C10,C01,C50 |  |

| Model Name: Haupt-Sedlan | Source: (Haupt; Sedlan, 2001) |
| --- | --- |
| Equation: W=C10I1-3+C01I2-3+C11I1-3I2-3+C02I2-32+C30I1-33 |  |
| Parameters: C10,C01,C11,C02,C30 |  |

| Model Name: Hartmann-Neff | Source: (Hartmann; Neff, 2003) |
| --- | --- |
| Equation: W=αI13-33+∑i=1N Ci0I1-3i+∑j=1N C0jI23/2-33j,N=1,2,3… |  |
| Parameters: α,Ci0,C0j |  |

| Model Name: Carroll | Source: (Carroll, 2011) |
| --- | --- |
| Equation: W=AI1+BI14+CI21/2 |  |
| Parameters: A, B, C |  |

| Model Name: Nunes | Source: (Nunes, 2011) |
| --- | --- |
| Equation: W=C1I1-3+43C2I2-33/4 |  |
| Parameters: C1,C2 |  |

| Model Name: Bahreman-Darijani | Source: (Bahreman; Darijani, 2015) |
| --- | --- |
| Equation: W=A2I1-3+B2I2-3+A4I12-2I2-3+A6I13-3I1I2 |  |
| Parameters: A2,B2,A4,A6 |  |

| Model Name: Zhao | Source: (Zhao; Mu; Du, 2019) |
| --- | --- |
| Equation: W=C-11I2-3+C11I1-3+C21I12-2I2-3+C22I12-2I2-32 |  |
| Parameters: C-11,C11,C21,C22 |  |

| Model Name: Arnoux | Source: (Arnoux, 2000; Arnoux et al., 2002) |
| --- | --- |
| Equation: W=c1expc2I1-3-c1c22I2-3 |  |
| Parameters: c1,c2 |  |

---

| Model Name: Warner | Source: (Warner, 1972) |
| --- | --- |
| Equation: W=-12μImln⁡1-I1-3Im-3 |  |
| Parameters: μ,Im |  |

| Model Name: Kilian | Source: (Kilian, 1981) |
| --- | --- |
| Equation: W=-μJLln⁡1-I1-3JL+I1-3JL |  |
| Parameters: μ,JL |  |

| Model Name: Van der Waals | Source: (Kilian, 1980) |
| --- | --- |
| Equation: W=μ-λm2-3[ln⁡(1-Θ)+Θ]-23aI-3232 |  |
| Parameters: μ,λm,β,a |  |

| Model Name: Gent | Source: (Gent, 1996) |
| --- | --- |
| Equation: W=-E6Im-3ln⁡1-I1-3Im-3 |  |
| Parameters: E,Im |  |

| Model Name: Takamizawa-Hayashi | Source: (Takamizawa; Hayashi, 1987) |
| --- | --- |
| Equation: W=-cln⁡1-I1-2Jm2 |  |
| Parameters: c,Jm |  |

| Model Name: Yeoh-Fleming | Source: (Yeoh; Fleming, 1997) |
| --- | --- |
| Equation: W=ABIm-31-e-BI1-3Im-3-C10Im-3ln⁡1-I1-3Im-3 |  |
| Parameters: A, B, Im,C10 |  |

| Model Name: 3 Parameters Gent | Source: (Gent, 1999) |
| --- | --- |
| Equation: W=μ2-αIm-3ln⁡1-I1-3Im-3+(1-α)I2-3 |  |
| Parameters: α,μ,Im |  |

| Model Name: Pucci-Saccomandi | Source: (Pucci; Saccomandi, 2002) |
| --- | --- |
| Equation: W=Kln⁡I23-μ2Jmln⁡1-I1-3Jm |  |
| Parameters: K,μ,Jm |  |

| Model Name: Horgan-Saccomandi | Source: (Horgan; Saccomandi, 2004) |
| --- | --- |
| Equation: W=-μ2Jln⁡J2-J2I1+JI2-1(J-1)3 |  |
| Parameters: μ,J |  |

| Model Name: Beatty | Source: (Beatty, 2008) |
| --- | --- |
| Equation: W=-cImIm-32Im-3ln⁡1-I1-3/Im-31+I1-3/Im |  |
| Parameters: c,Im |  |

| Model Name: Horgan-Murphy | Source: (Horgan; Murphy, 2007) |
| --- | --- |
| Equation: W=-2μIm-3c2ln⁡1-λ1c+λ2c+λ3c-3Im-3 |  |
| Parameters: μ,c,Im |  |

| Model Name: bin Othman & Gregory | Source: (Othman; Gregory, 1990) |
| --- | --- |
| Equation: A2I1-3-CB2log⁡BCI1-3+1+1BI1-3 |  |
| Parameters: A, B, C |  |

---

| Model Name: Knowles | Source: (Knowles, 1977) |
| --- | --- |
| Equation: W=μ2b1+bI1-3nn-1 |  |
| Parameters: μ,b,n |  |

| Model Name: Swanson | Source: (Swanson, 1985) |
| --- | --- |
| Equation: W=32∑i=1N Ai1+αiI131+αi+32∑i=1N Bi1+βiI231+βi,N=1,2,3… |  |
| Parameters: Ai,αi,Bi,βi |  |

| Model Name: Yamashita-Kawabata | Source: (Yamashita; Kawabata, 1992) |
| --- | --- |
| Equation: W=C1I1-3+C2I2-3+C3N+1I1-3N+1 |  |
| Parameters: C1,C2,C3,N |  |

| Model Name: Davis-De-Thomas | Source: (Davies; De; Thomas, 1994) |
| --- | --- |
| Equation: W=A2(1-n/2)I1-3+C2(1-n/2)+kI1-32 |  |
| Parameters: A, n, C, k |  |

| Model Name: Gregory | Source: (Gregory; Muhr; Stephens, 1997) |
| --- | --- |
| Equation: W=A2(1-n/2)I1-3+C2(1-n/2)+B2(1+m/2)I1-3+C2(1+m/2) |  |
| Parameters: A, B, C, m, n |  |

| Model Name: modified Gregory | Source: (He et al., 2022) |
| --- | --- |
| Equation: W=A1+αI1-3+M21+α+B1+βI1-3+N21+β |  |
| Parameters: A,α,M,B,β,N |  |

| Model Name: modified Gregory | Source: (He et al., 2022) |
| --- | --- |
| Equation: W=A1+αI1-3+M21+α+B1+βI1-3+N21+β |  |
| Parameters: A,α,M,B,β,N |  |

| Model Name: Beda | Source: (Beda, 2005) |
| --- | --- |
| Equation: W=C1αI1-3α+C2I1-3+C3ςI1-3ς+K0βI2-3β |  |
| Parameters: C1,C2,C3,α,ς,β,K0 |  |

| Model Name: Amin | Source: (Amin et al., 2006) |
| --- | --- |
| Equation: W=C1I1-3+C2N+1I1-3N+1+C3M+1I1-3M+1+C4I2-3 |  |
| Parameters: C1,C2,C3,C4,N,M |  |

| Model Name: Lopez-Pamies | Source: (Lopez-Pamies, 2010) |
| --- | --- |
| Equation: W=∑i=1N 31-αiαiμiI1αi-3αi,N=1,2,3… |  |
| Parameters: αi,μi |  |

| Model Name: gen-Yeoh | Source: (Hohenberger et al., 2019) |
| --- | --- |
| Equation: W=K1I1-3m+K2I1-3p+K3I1-3q |  |
| Parameters: K1,K2,K3,m,p,q |  |

| Model Name: Hart-Smith | Source: (Hart-Smith, 1966) |
| --- | --- |
| Equation: ∂W∂I1=Gexp⁡k1I1-32,∂W∂I2=Gk2I2 |  |
| Parameters: G,k1,k2 |  |

| Model Name: Veronda-Westmann | Source: (Veronda; Westmann, 1970) |
| --- | --- |
| Equation: W=C1eαI1-3-1+C2I2-3 |  |
| Parameters: C1,C2,α |  |

| Model Name: Fung-Demiray | Source: (Demiray, 1972; Fung, 1967) |
| --- | --- |
| Equation: W=μ2bebI1-3-1 |  |
| Parameters: μ,b |  |

| Model Name: Vito | Source: (Vito, 1973) |
| --- | --- |
| Equation: W=μ2bebαI1-3+(1-α)I2-3-1 |  |
| Parameters: μ,b,α |  |

| Model Name: Humphrey-Yin | Source: (Humphrey; Yin, 1987) |
| --- | --- |
| Equation: W=C1eC2I1-3-1 |  |
| Parameters: C1,C2 |  |

| Model Name: modified Yeoh | Source: (Yeoh, 1993) |
| --- | --- |
| Equation: W=C10I1-3+C20I1-32+C30I1-33+αβ1-e-βI1-3 |  |
| Parameters: C10,C20,C30,α,β |  |

| Model Name: Martins | Source: (Martins et al., 1998) |
| --- | --- |
| Equation: W=C1eC2I1-3-1+C3eC4λf-12-1 |  |
| Parameters: C1,C2,C3,C4 |  |

| Model Name: Chevalier-Marco | Source: (Gornet et al., 2012) |
| --- | --- |
| Equation: W=h1∫eh3I1-32dI1+3h2∫1I2dI2 |  |
| Parameters: h1,h2,h3 |  |

| Model Name: Mansouri-Darijani | Source: (Mansouri; Darijani, 2014) |
| --- | --- |
| Equation: W=A1em1I1-3-1+B1en1I2-3-1 |  |
| Parameters: A1,m1,B1,n1 |  |

| Model Name: Gent-Thomas | Source: (Gent; Thomas, 1958) |
| --- | --- |
| Equation: W=C1I1-3+C2ln⁡I23 |  |
| Parameters: C1,C2 |  |

| Model Name: Alexander | Source: (Alexander, 1968) |
| --- | --- |
| Equation: W=C1∫exp⁡kI1-32dI1+C2ln⁡I2-3+γγ+C3I2-3 |  |
| Parameters: C1,k,C2,γ,C3 |  |

| Model Name: Lambert Diani-Rey | Source: (Lambert-Diani; Rey, 1999) |
| --- | --- |
| Equation: W=∫exp⁡∑i=02 aiI1-3idI1+∫exp⁡∑i=01 biln⁡I2idI2 |  |
| Parameters: a0,a1,a2,b0,b1 |  |

| Model Name: Hoss-Marczak-I | Source: (Hoss; Marczak, 2010) |
| --- | --- |
| Equation: W=αβ1-e-βI1-3+μ2b1+bI1-3nn-1 |  |
| Parameters: α,β,μ,b,n |  |

| Model Name: Hoss-Marczak-II | Source: (Hoss; Marczak, 2010) |
| --- | --- |
| Equation: W=αβ1-e-βI1-3+μ2b1+bI1-3nn-1+C2ln⁡I23 |  |
| Parameters: α,β,μ,b,n,C2 |  |

| Model Name: Exp-Ln | Source: (Khajehsaeid; Arghavani; Naghdabadi, 2013) |
| --- | --- |
| Equation: W=A1aeaI1-3+bI1-21-ln⁡I1-2-1a-b |  |
| Parameters: A,a,b |  |

| Model Name: modified Demiray | Source: (Demiray et al., 1988) |
| --- | --- |
| Equation: W=α4I1-32+β4cexp⁡cI1-32-1 |  |
| Parameters: α, β, c |  |

| Model Name: Holmes-Mow | Source: (Holmes; Mow, 1990) |
| --- | --- |
| Equation: W=c0exp⁡c1I1-3+exp⁡c2I2-3-c0 |  |
| Parameters: c0, c1, c2 |  |

| Model Name: Raghavan-Vorp | Source: (Raghavan; Vorp, 2000) |
| --- | --- |
| Equation: W=c1I1-3+c2I1-32 |  |
| Parameters: c1, c2 |  |

| Model Name: Singh | Source: (Singh; Katiyar; Singh, 2013) |
| --- | --- |
| Equation: W=C12bexp⁡bI1-3-1+C22I2-32 |  |
| Parameters: C1, C2, b |  |

| Model Name: Tang | Source: (Tang et al., 2009) |
| --- | --- |
| Equation: W=c1I1-3+c2I2-3+D1exp⁡D2I1-3-1 |  |
| Parameters: c1, c2, D1, D2 |  |

| Model Name: van Dam | Source: (Van Dam et al., 2008) |
| --- | --- |
| Equation: W=G0-1-AC2Cx+1exp⁡-Cx-1+12Ax2 with x=c4I1+1-c4I2-3 |  |
| Parameters: A,b, C |  |

| Model Name: modified exp-ln | Source: (Dwivedi et al., 2022) |
| --- | --- |
| Equation: W=-A1aexp⁡-aI1-3-1+bI1-21-ln⁡I1-2-1-1a-b |  |
| Parameters: G0,a, b |  |

| Model Name: Fu | Source: (Van Dam et al., 2008) |
| --- | --- |
| Equation: G12I1-3+120NI12-9+111050N2I13-27+197000N3I14-81+ 519673750N4I15-243+abexp⁡bI1-3-1 |  |
| Parameters: G, N, a, b |  |

#### Mixed Phenomenological Model

| Model Name: Continuum Hybrid | Source: (Beda; Chevalier, 2003) |
| --- | --- |
| Equation: W=K1I1-3+K2ln⁡I23+μαλ1α+λ2α+λ3α-3 |  |
| Parameters: K1,K2,α,μ |  |

| Model Name: Bechir-4term | Source: (Bechir et al., 2006) |
| --- | --- |
| Equation: W=C11I1-3+∑n=12 ∑r=12 Cnrλ12n+λ22n+λ32n-3r |  |
| Parameters: C11,C12,C21,C22 |  |

| Model Name: WFB | Source: (Korba; Barkey, 2017) |
| --- | --- |
| Equation: W=∫1Lf Fλ1Aλ1e-BI1+Cλ1I1-Dλ12-1λ1dλ1 |  |
| Parameters: A, B, C, D |  |

| Model Name: Volokh-Vorp | Source: (Volokh; Vorp, 2008) |
| --- | --- |
| Equation: W=φ-φexp⁡-αφI1-3-βφI1-32 |  |
| Parameters: φ,α, β |  |

### Micromechanical Network Models

Micromechanical network models are based on the statistical mechanics of polymer chains, relating macroscopic behavior to the material’s underlying microstructure. These models provide a physical interpretation of parameters and are especially useful when microstructural insights are important for design or analysis.

#### Gaussian Chain Network Models

| Model Name: Gaussian | Source: (Treloar, 1943a) |
| --- | --- |
| Equation: W=12NkTλ12+λ22+λ32-3 |  |
| Parameters: NkT |  |

| Model Name: Affine | Source: (Flory, 1976; Kuhn, 1946; Wall; Flory, 1951) |
| --- | --- |
| Equation: W=G2λ12+λ22+λ32-3,G=vkT |  |
| Parameters: G |  |

| Model Name: Phantom | Source: (Flory, 1976; Kuhn, 1946; Wall; Flory, 1951) |
| --- | --- |
| Equation: W=vkT21-2fλ12+λ22+λ32-3,Gc=vkT(1-2/f) |  |
| Parameters: Gc |  |

| Model Name: Edwards-Tube | Source: (Edwards, 1967) |
| --- | --- |
| Equation: W=Geλ12+λ22+λ32-3 |  |
| Parameters: Ge |  |

| Model Name: Slip-Link | Source: (Ball et al., 1981) |
| --- | --- |
| Equation: W=12Ge∑i=13 (1+η)1-α2λi21+ηλi21-α2∑i=13 λi2+ln⁡1+η∑i=13 λi2 |  |
| Parameters: Ge,η,α |  |

| Model Name: Constrained Junctions | Source: (Erman; Flory, 1982; Flory; Erman, 1982) |
| --- | --- |
| Equation: W=Wph+νkT2∑t κλt2-1λt2+κ+ln⁡λt2+κ1+κ-ln⁡λt2 |  |
| Parameters: Gc,νkT,κ |  |

| Model Name: Edwards-Vilgis | Source: (Edwards; Vilgis, 1986) |
| --- | --- |
| Equation: W=Gc21-α2I11-α2I1+ln⁡1-α2I1+12Ge∑i=13 (1+η)1-α2λi21+ηλi21-α2∑i=13 λi2+ln⁡1+η∑i=13 λi2 |  |
| Parameters: Gc,Ge,α,η |  |

| Model Name: MCC | Source: (Erman; Monnerie, 1989) |
| --- | --- |
| Equation: W=12ξkT∑i λi2-1+12μkT∑i Bt+Dt-ln⁡1+Bt-ln⁡1+Dt |  |
| Parameters: ξkT,μkT,κ |  |

| Model Name: Tube | Source: (Heinrich; Kaliske, 1997) |
| --- | --- |
| Equation: Bt =κ2λt2-1λt2+κ-2,Dt=λt2Bt/κW =i=13 Gc2λi2-1+2Geβ2λi-β-1 |  |
| Parameters: Gc,Ge,β |  |

| Model Name: Nonaffine-Tube | Source: (Rubinstein; Panyukov, 1997) |
| --- | --- |
| Equation: W=Wph+Went =Gc∑i=13 λi22+Ge∑i=13 λi+1λi |  |
| Parameters: Gc,Ge |  |

#### Non-Gaussian Chain Network Models

| Model Name: Three-Chain | Source: (James; Guth, 1943) |
| --- | --- |
| Equation: W=μN3∑i=13 N-1λiβi+ln⁡βisinh⁡βi |  |
| Parameters: μ,N |  |

| Model Name: Four-Chain | Source: (Flory, 1944) |
| --- | --- |
| Equation: W=μN3∑i=13 N-1λiβi+ln⁡βisinh⁡βi |  |
| Parameters: μ,N |  |

| Model Name: Arruda-Boyce | Source: (Arruda; Boyce, 1993; Cohen, 1991) |
| --- | --- |
| Equation: W=μNβchain λchain +ln⁡βchain sinh⁡βchain ;W=G16I1-3-Im3ln⁡1-I1-3Im-3 |  |
| Parameters: μ,N-G,Im |  |

| Model Name: modified Flory-Erman | Source: (Boyce; Arruda, 2000; Edwards, 1967) |
| --- | --- |
| Equation: W =W8ch+i=13 μ2Bi+Di-ln⁡Bi+1-ln⁡Di+1Bt =κ2λt2-1λt2+κ-2Dt=λt2Bt/κ |  |
| Parameters: μ,N,κ |  |

| Model Name: Extended-Tube | Source: (Kaliske; Heinrich, 1999) |
| --- | --- |
| Equation: W=Gc21-δ2I1-31-δ2I1-3+ln⁡1-δ2I1-3+2Geβ2∑i=13 λi-β-1 |  |
| Parameters: Gc,Ge,δ,β |  |

| Model Name: Meissner-Matějka | Source: (Meissner; Matějka, 2003, 2004) |
| --- | --- |
| Equation: W=μNγλc,r+ln⁡γsinh⁡γ+∑i=13 2μeβ2λi-β-1 |  |
| Parameters: μ,N,μe,β |  |

| Model Name: Micro-Sphere | Source: (Miehe; Göktepe; Lulei, 2004) |
| --- | --- |
| Equation: W=μλrL-1λr+ln⁡L-1λrsinh⁡L-1λr |  |
| Parameters: μ,N,P,U,q |  |

| Model Name: Bootstrapped-8-Chain | Source: (Miroshnychenko; Green, 2009; Miroshnychenko; Green; Turner, 2005) |
| --- | --- |
| Equation: W=μλrL-1λr+ln⁡L-1λrsinh⁡L-1λr |  |
| Parameters: μ,N,P,U,q |  |

| Model Name: Davidson-Goulbourne | Source: (Davidson; Goulbourne, 2013) |
| --- | --- |
| Equation: W=16GcI1-Gcλmax 2ln⁡3λmax 2-I1+Ge∑i λi+1λi |  |
| Parameters: Gc,Ge,λmax |  |

| Model Name: Network Averaging Tube | Source: (Khiêm; Itskov, 2016) |
| --- | --- |
| Equation: W=μcκnln⁡sin⁡πnI13q2sin⁡πnI13q2+μtI2312-1 |  |
| Parameters: μcκ,n,q,μt |  |

| Model Name: SpT | Source: (Xiang et al., 2018) |
| --- | --- |
| Equation: W=GcNln⁡3N+12I13N-I1+Ge∑i 1λi |  |
| Parameters: Gc,Ge,N |  |

| Model Name: Dobrynin & Carrillo | Source: (Dobrynin; Carrillo, 2011) |
| --- | --- |
| Equation: G2I1-3+6I1-3(β-1)I1-3 |  |
| Parameters: G, β |  |

| Model Name: Simplified Tube | Source: (Plagge et al., 2020) |
| --- | --- |
| Equation: Gc2I1-31-I1-3/n |  |
| Parameters: GC, n |  |

#### Mixed Micromechanical Network Models

| Model Name: Wu-Giessen (Full Network) | Source: (Treloar; Riding, 1997; Wu; Van der Giessen, 1992; Wu; Van Der Giessen, 1993) |
| --- | --- |
| Equation: W=μ3N∑i=13 λiβi+Nln⁡βisinh⁡βi(1-ρ)+μNλchain βchain +ln⁡βchain sinh⁡βchain ρ |  |
| Parameters: μ,N,ρ |  |

| Model Name: Zuniga-Beatty | Source: (Elı́as-Zúñiga; Beatty, 2002) |
| --- | --- |
| Equation: W=W3ch1-λ1N3+W8chλ12+λ22+λ323N8 |  |
| Parameters: μ,N3,N8 |  |

| Model Name: Lim | Source: (Elı́as-Zúñiga; Beatty, 2002) |
| --- | --- |
| Equation: W=WGaussian (1-f)+fW8ch |  |
| Parameters: NkT, μ,N,f |  |

| Model Name: Lim | Source: (Lim, [S.d.]) |
| --- | --- |
| Equation: W=WGaussian (1-f)+fW8ch |  |
| Parameters: NkT, μ,N,f |  |

| Model Name: Bechir-Chevalier | Source: (Bechir; Chevalier; Idjeri, 2010) |
| --- | --- |
| Equation: W=μfN8λrβ+ln⁡βsinh⁡β+μc3N3∑j=1j=3 βjλjr+ln⁡βjsinh⁡βj |  |
| Parameters: μ0,η,ρ,N3,N8 |  |

| Model Name: Davies | Source: (Bechir; Chevalier; Idjeri, 2010) |
| --- | --- |
| Equation: W=μfN8λrβ+ln⁡βsinh⁡β+μc3N3∑j=1j=3 βjλjr+ln⁡βjsinh⁡βj |  |
| Parameters: μ0,η,ρ,N3,N8 |  |

### Hookean-type Models

Every Hookean-type hyperelastic model have mechanical behavior intrinsically linked to the used strain measure. The linear relationship between the conjugated pair of stress and strain, as well as the Strain Energy Function the represent the hyperelastic model are presented below. As one can see, every Hookean-type model has at least two material parameters: Young’s modulus (E) and Poisson’s ratio (ν), which are used to define the Lamé Parameters (L1 and L2), so any other parameter necessary to define the strain measure is added to those two.

σ=L1trεI+2L2ε     ou     σij=L1εkkδij+2L2εij

ψε=L12trε2+L2trε2     ou     ψεij=L12εkk2+L2εlmεml

L1=E.ν1+ν.1-2.ν

L2=E2.1+ν

| Strain Measure: Generalized Hyperbolic-Sine (GHS) | Source: (Peixoto, 2024; Peixoto; Greco; Vasconcellos, 2024) |
| --- | --- |
| Equation: ελ=n-1sinhnfλ or εU=n-1sinhnfU n≠0 *fλ and fU can be any given strain measure in its uniaxial and multiaxial format, respectively. |  |
| Parameters: E, ν, n |  |

| Strain Measure: Seth-Hill (Doyle-Ericken or Classical) | Source: (Doyle; Ericksen, 1956; Hill, 1968; Seth, 1961) |
| --- | --- |
| Equation: ε=1mλm-1 para m≠0lnλ para m=0 ε=1mUm-1 para m≠0lnU para m=0 |  |
| Parameters: E, ν, m |  |

| Strain Measure: GHS–Seth-Hill | Source: (Peixoto, 2024; Peixoto; Greco; Vasconcellos, 2024) |
| --- | --- |
| Equation: ε=n-1senhnmλm-1 para m≠0n-1senhnlnλ=12nλn-λ-n para m=0 ε=n-1senhnmUm-1 para m≠0n-1senhnlnU para m=0 |  |
| Parameters: E, ν,n, m |  |

| Strain Measure: Curnier-Zysset Metric Family | Source: (Curnier; Zysset, 2006) |
| --- | --- |
| Equation: ε=2+n8C-n4I-2-n8C-1; ε=O se U=C=I ; (-2⩽n⩽2) |  |
| Parameters: E, ν,n |  |

| Strain Measure: Curnier-Rakotomanana Family (Darijani-Naghdabadi Power Family) | Source: (Curnier; Rakotomanana, 1991; Darijani; Naghdabadi, 2013) |
| --- | --- |
| Equation: ε=1m+nUm-U-n; mn>0 or m,n=0 |  |
| Parameters: E, ν,m,n |  |

| Strain Measure: Darijani-Naghdabadi Exponencial Family | Source: (Darijani; Naghdabadi, 2013) |
| --- | --- |
| Equation: ε=1m+nemU-I-enU-1-I; mn>0 or m,n=0 |  |
| Parameters: E, ν,m,n |  |

| Strain Measure: Beex Family | Source: (Beex, 2019) |
| --- | --- |
| Equation: ε=m ∈ Asmεm ; m ∈ Asm=1 ; * A is a set: A=m1,m2, …, mn and εm are the Seth-Hill strain measures. |  |
| Parameters: E, ν,n, m,s |  |

## Use of Hyperelastic Models in HyperSmart

In HyperSmart, hyperelastic models are stored in YAML (.yaml) files, a format chosen for its readability, flexibility, and ease of editing. This structure allows the program to load model definitions dynamically and enables users to expand the model library without modifying the source code.

Users can add new models from the literature—or create their own—by preparing a .yaml file following the same structure as the existing examples. To derive the mathematical expressions, the formulae presented in the “2.1 Pure Deformation Modes Expressions” subsection can be used.

A model .yaml file contains:

- Basic Information: model_name, model_class, and description, summarizing the model and its category.

- References: Complete bibliographic citations for traceability.

- Mathematical Definition: The strain energy function or strain measure, depending on the model type.

- Parameters: Number of material parameters, their names, display formats, and bounds.

- Deformation Mode Expressions: Analytical formulas for each pure deformation mode (uniaxial, biaxial, pure shear, and simple shear) in both nominal and Cauchy stress forms, when applicable.

- Additional Information: Optional constraints or notes relevant to the model.

Example – Mooney–Rivlin Model

When a .yaml model file is added to the appropriate directory, HyperSmart automatically recognizes it and makes it available for selection in the program. By following this structure, users can:

- Integrate models from the literature into the software’s library.

- Implement custom models for specialized applications.

- Share model files with other researchers, ensuring reproducibility and consistency.

# Numerical Methods of Calibration

This section describes how numerical methods are implemented in HyperSmart to calibrate hyperelastic material parameters from experimental data, and explain the extensible architecture used that makes it easy to add new methods. Rather than hard-coding solvers, the application exposes a plugin-style interface: each numerical method is defined by a pair of files in numerical_methods/ folder: a small YAML manifest (name + description) and a Python module that exports a unified MethodWindow(...) class. At runtime the app discovers methods dynamically by scanning the YAML files, lists them in the UI, and loads the corresponding Python module by stem (no changes to the main program). This separation of concerns – core UI/navigation vs. self-contained method plugins – lets users add, replace, or compare methods simply by dropping in new files (e.g., our current Enumeration and BUS implementations), ensuring the system can grow without refactoring the application core.

## Calibration methods

Below the two methods already avaiable in the program are presented.

### Enumeration (grid search)

Enumeration performs a brute-force search over a user-defined grid in parameter space. For each material parameter θj​, the user specifies a lower bound, an upper bound, and a step size Δj​. The algorithm evaluates the model on the Cartesian product of these grids and returns the parameter set that minimizes the error metric.

- Strengths: simple, transparent, and guarantees the exact optimum if it lies on the grid. Useful for low-dimensional problems, coarse sweeps, and diagnostics.

- Limitations: computational cost grows exponentially with the number of parameters and with finer step sizes.

### BUS (Bayesian Updating with Subset Simulation)

BUS casts calibration as Bayesian inference. Starting from user-defined priors on the parameters, it constructs an augmented probability space and uses Subset Simulation to sample progressively from higher-likelihood regions until reaching the posterior. It returns:

- a MAP (maximum a posteriori) estimate, and

- a sampled posterior for uncertainty quantification and parameter correlations.

- Strengths: uncertainty quantification; often more efficient than plain MCMC for multi-modal or high-dimensional posteriors; natural credibility intervals.

- Limitations: require evaluating the forward model at every sample; results depend on the choice of priors, noise model (likelihood), and BUS hyperparameters (samples per level N, target conditional probability p0​).

## Error metric and multi-mode calibration

We use a normalized root-mean-square error tailored for simultaneous calibration across multiple deformation modes:

REQMN=1Pi=1P σiexp-σimodel 21Pi=1P σiexp

where P is the total number of experimental points across all selected modes. The normalization by the mean absolute experimental stress prevents modes with larger stress magnitudes from dominating the fit. REQMN is well-defined even when stresses include negative values; only the (rare) case of a near-zero denominator requires caution.

- Enumeration Method minimizes REQMN over the grid.

- BUS uses a Gaussian likelihood on residuals (equivalent to minimizing the sum of squared errors) and reports REQMN for diagnostics.

## Data handling and model evaluation

Experimental data are carried in a single ExperimentalData object:

- deformation-mode arrays (e.g., uniaxial, biaxial, pure shear, simple shear)

- stress measure flag (nominal vs. Cauchy)

- unit metadata (MPa/kPa)

- optional pre-assigned constants

For each selected mode, the program maps experimental independent variables to the symbolic names used by the model expressions (e.g., lamb for stretch, gamma for shear). The stress measure is respected by selecting the matching expression (e.g., expression_nominal or expression_cauchy).

Some models include conditional expressions (e.g., the Generalized Hyperbolic Sine Hookean-tye model has one formula for m≠0 and another for m=0). Each model’s YAML can therefore provide either a single string formula or a list of {condition, formula} entries. A small evaluator selects the formula whose condition is true for the current parameter values, ensuring the correct branch is used during calibration.

## Plugin architecture for numerical methods

To enable new methods without changes to the main code, we use a lightweight plugin pattern:

- Each method lives in two files inside src/numerical_methods/:

  - a YAML metadata file: method_stem.yaml

  - a Python implementation: method_stem.py

- The YAML must define:

  - method_name – display name shown in the UI

  - description – short explanation shown in the info panel

- The Python module must expose a class named MethodWindow with the unified constructor:

“MethodWindow(root, material, model_data, proceed_callback_back)”

This class is responsible for rendering its own UI (bounds/steps for Enumeration; priors and 	BUS settings for BUS) and for running the method.

- The application discovers methods dynamically:

- It scans numerical_methods/*.yaml, displays their method_name in the selection window, and remembers each file’s stem (e.g., enumeration).

- When the user clicks Next, the app imports numerical_methods.<stem> and instantiates MethodWindow(...). No changes to the main script are needed to support new methods.

Benefits of this design

- Extensibility: users can add new calibration strategies by dropping in a .yaml + .py pair; the main app remains untouched.

- Separation of concerns: the core UI manages discovery and navigation; each method owns its own inputs, solver, and result reporting.

- Consistency: all methods adhere to the same constructor, making the user experience uniform.

- Reproducibility & transparency: the YAML summaries document method intent and options; Enumeration grids and BUS priors/hyperparameters are all explicit.

## Typical Workflow

1 	Choose a model and review its description/equations.

2 	Pick a calibration method:

- Enumeration: set parameter ranges and step sizes.

- BUS: set priors, noise level σ, samples/level N, and p0​.

3 	Select deformation modes to include (one or multiple).

4 	Run calibration:

- Enumeration searches the grid and reports the parameter set with the lowest REQMN.

- BUS estimates the posterior, reports MAP parameters, and provides posterior samples for uncertainty analysis.

5 	Inspect results: calibrated parameters, REQMN, and plots comparing experimental vs. model stress for each mode.

# Referências

ALEXANDER, H. A constitutive relation for rubber-like materials. International Journal of Engineering Science, v. 6, n. 9, p. 549–563, 1 set. 1968.

AMIN, A. F. M. S. et al. Hyperelasticity model for finite element analysis of natural and high damping rubbers in compression and shear. Journal of Engineering Mechanics, v. 132, n. 1, p. 54–64, 2006.

ARNOUX, P. J. Modélisation des ligaments des membres porteurs. [S.l.]: Université de la Méditerranée, 2000.

ARNOUX, P. J. et al. A Visco-hyperelastic Model With Damage for the Knee Ligaments Under Dynamic Constraints. Computer Methods in Biomechanics and Biomedical Engineering, v. 5, n. 2, p. 167–174, 1 jan. 2002.

ARRUDA, Ellen M.; BOYCE, Mary C. A three-dimensional constitutive model for the large stretch behavior of rubber elastic materials. Journal of the Mechanics and Physics of Solids, v. 41, n. 2, p. 389–412, 1 fev. 1993.

ATTARD, Mario M.; HUNT, Giles W. Hyperelastic constitutive modeling under finite strain. International Journal of Solids and Structures, v. 41, n. 18, p. 5327–5350, 1 set. 2004.

BAHREMAN, Marzieh; DARIJANI, Hossein. New polynomial strain energy function; application to rubbery circular cylinders under finite extension and torsion. Journal of Applied Polymer Science, v. 132, n. 13, 2015.

BALL, R. C. et al. Elasticity of entangled networks. Polymer, v. 22, n. 8, p. 1010–1018, 1 ago. 1981.

BEATTY, Millard F. On Constitutive Models for Limited Elastic, Molecular Based Materials. Mathematics and Mechanics of Solids, v. 13, n. 5, p. 375–387, 1 jul. 2008.

BECHIR, H. et al. Hyperelastic constitutive model for rubber-like materials based on the first Seth strain measures invariant. European Journal of Mechanics - A/Solids, v. 25, n. 1, p. 110–124, 1 jan. 2006.

BECHIR, H.; CHEVALIER, L.; IDJERI, M. A three-dimensional network model for rubber elasticity: The effect of local entanglements constraints. International Journal of Engineering Science, v. 48, n. 3, p. 265–274, 1 mar. 2010.

BEDA, T. Reconciling the fundamental phenomenological expression of the strain energy of rubber with established experimental facts. Journal of Polymer Science Part B: Polymer Physics, v. 43, n. 2, p. 125–134, 2005.

BEDA, T.; CHEVALIER, Y. Hybrid continuum model for large elastic deformation of rubber. Journal of Applied Physics, v. 94, n. 4, p. 2701–2706, 2003.

BEEX, L. A. A. Fusing the Seth–Hill strain tensors to fit compressible elastic material responses in the nonlinear regime. International Journal of Mechanical Sciences, v. 163, p. 105072, 1 nov. 2019.

BIDERMAN, V. L. Calculation of rubber parts. Rascheti na prochnost, v. 40, 1958.

BOYCE, Mary C.; ARRUDA, Ellen M. Constitutive Models of Rubber Elasticity: A Review. Rubber Chemistry and Technology, v. 73, n. 3, p. 504–523, 1 jul. 2000.

CARROLL, M. M. A Strain Energy Function for Vulcanized Rubbers. Journal of Elasticity, v. 103, n. 2, p. 173–187, 1 abr. 2011.

COHEN, A. A Padé approximant to the inverse Langevin function. Rheologica Acta, v. 30, n. 3, p. 270–273, 1 maio 1991.

CURNIER, A.; RAKOTOMANANA, L. Generalized Strain and Stress Measures: Critical Survey and New Results. Engineering Transactions, v. 39, n. 3–4, p. 461–538, 31 dez. 1991.

CURNIER, A.; ZYSSET, Ph. A family of metric strains and conjugate stresses, prolonging usual material laws from small to large transformations. International Journal of Solids and Structures, v. 43, n. 10, p. 3057–3086, 1 maio 2006.

DARIJANI, H.; NAGHDABADI, R. Kinematics and kinetics modeling of thermoelastic continua based on the multiplicative decomposition of the deformation gradient. International Journal of Engineering Science, v. 62, p. 56–69, 1 jan. 2013.

DAVIDSON, Jacob D.; GOULBOURNE, N. C. A nonaffine network model for elastomers undergoing finite deformations. Journal of the Mechanics and Physics of Solids, v. 61, n. 8, p. 1784–1797, 1 ago. 2013.

DAVIES, C. K. L.; DE, Dilip K.; THOMAS, A. G. Characterization of the Behavior of Rubber for Engineering Design Purposes. 1. Stress-Strain Relations. Rubber Chemistry and Technology, v. 67, n. 4, p. 716–728, 1 set. 1994.

DEMIRAY, Hilmi. A note on the elasticity of soft biological tissues. Journal of Biomechanics, v. 5, n. 3, p. 309–311, 1 maio 1972.

DEMIRAY, Hilmi et al. A stress-strain relation for a rat abdominal aorta. Journal of Biomechanics, v. 21, n. 5, p. 369–374, 1 jan. 1988.

DOBRYNIN, Andrey V.; CARRILLO, Jan-Michael Y. Universality in Nonlinear Elasticity of Biological and Polymeric Networks and Gels. Macromolecules, v. 44, n. 1, p. 140–146, 11 jan. 2011.

DOYLE, T. C.; ERICKSEN, Jerald L. NONLINEAR ELASTICITY. [S.l.]: ACADEMIC PRESS, 1956.

DWIVEDI, Krashn Kr. et al. A hyperelastic model to capture the mechanical behaviour and histological aspects of the soft tissues. Journal of the Mechanical Behavior of Biomedical Materials, v. 126, p. 105013, 1 fev. 2022.

EDWARDS, S. F. The statistical mechanics of polymerized material. Proceedings of the Physical Society, v. 92, n. 1, p. 9–16, 1967.

EDWARDS, S. F.; VILGIS, Th. The effect of entanglements in rubber elasticity. Polymer, v. 27, n. 4, p. 483–492, 1 abr. 1986.

ELÍAS-ZÚÑIGA, Alex; BEATTY, Millard F. Constitutive equations for amended non-Gaussian network models of rubber elasticity. International Journal of Engineering Science, v. 40, n. 20, p. 2265–2294, 1 dez. 2002.

ERMAN, Burak; FLORY, Paul J. Relationships between stress, strain, and molecular constitution of polymer networks. Comparison of theory with experiments. Macromolecules, v. 15, n. 3, p. 806–811, 1 maio 1982.

ERMAN, Burak; MONNERIE, Lucien. Theory of elasticity of amorphous networks: effect of constraints along chains. Macromolecules, v. 22, n. 8, p. 3342–3348, 1 ago. 1989.

FLORY, P. J. STATISTICAL THERMODYNAMICS OF RANDOM NETWORKS. Proc R Soc London Ser A, v. 351, n. 1666, p. 351–380, 1976.

FLORY, Paul J. Network Structure and the Elastic Properties of Vulcanized Rubber. Chemical Reviews, v. 35, n. 1, p. 51–75, 1 ago. 1944.

FLORY, Paul J.; ERMAN, Burak. Theory of elasticity of polymer networks. 3. Macromolecules, v. 15, n. 3, p. 800–806, 1 maio 1982.

FUNG, Yc. Elasticity of soft tissues in simple elongation. American Journal of Physiology-Legacy Content, v. 213, n. 6, p. 1532–1544, dez. 1967.

GENT, A. N. A New Constitutive Relation for Rubber. Rubber Chemistry and Technology, v. 69, n. 1, p. 59–61, 1 mar. 1996.

GENT, A. N. Elastic Instabilities of Inflated Rubber Shells. Rubber Chemistry and Technology, v. 72, n. 2, p. 263–268, 1 maio 1999.

GENT, A. N.; THOMAS, A. G. Forms for the stored (strain) energy function for vulcanized rubber. Journal of Polymer Science, v. 28, n. 118, p. 625–628, 1958.

GORNET, L. et al. A new isotropic hyperelastic strain energy function in terms of invariants and its derivation into a pseudo-elastic model for Mullins effect: Application to finite element analysis. In: CONSTITUTIVE MODELS FOR RUBBER VII - PROCEEDINGS OF THE 7TH EUROPEAN CONFERENCE ON CONSTITUTIVE MODELS FOR RUBBER, ECCMR. Anais... 2012.

GREGORY, I. H.; MUHR, A. H.; STEPHENS, I. J. Engineering applications of rubber in simple extension. Plastics, Rubber and Composites Processing and Applications, v. 26, n. 3, p. 118–122, 1997.

HAINES, D. W.; WILSON, W. D. Strain-energy density function for rubberlike materials. Journal of the Mechanics and Physics of Solids, v. 27, n. 4, p. 345–360, 1 ago. 1979.

HARTMANN, Stefan; NEFF, Patrizio. Polyconvexity of generalized polynomial-type hyperelastic strain energy functions for near-incompressibility. International Journal of Solids and Structures, v. 40, n. 11, p. 2767–2791, 1 jun. 2003.

HART-SMITH, L. J. Elasticity parameters for finite deformations of rubber-like materials. Zeitschrift für angewandte Mathematik und Physik ZAMP, v. 17, n. 5, p. 608–626, 1966.

HAUPT, P.; SEDLAN, K. Viscoplasticity of elastomeric materials: Experimental facts and constitutive modelling. Archive of Applied Mechanics, v. 71, n. 2–3, p. 89–109, 2001.

HE, Hong et al. A comparative study of 85 hyperelastic constitutive models for both unfilled rubber and highly filled rubber nanocomposite material. Nano Materials Science, Trends in Nanomaterials and Nanocomposites: Fundamentals, Modelling and Applications - Part A. v. 4, n. 2, p. 64–82, 1 jun. 2022.

HEINRICH, G.; KALISKE, M. Theoretical and numerical formulation of a molecular based constitutive tube-model of rubber elasticity. Computational and Theoretical Polymer Science, v. 7, n. 3, p. 227–241, 1 jan. 1997.

HILL, R. On constitutive inequalities for simple materials—I. Journal of the Mechanics and Physics of Solids, v. 16, n. 4, p. 229–242, 1 ago. 1968.

HOHENBERGER, Travis W. et al. A CONSTITUTIVE MODEL FOR BOTH LOW AND HIGH STRAIN NONLINEARITIES IN HIGHLY FILLED ELASTOMERS AND IMPLEMENTATION WITH USER-DEFINED MATERIAL SUBROUTINES IN ABAQUS. Rubber Chemistry and Technology, v. 92, n. 4, p. 653–686, 1 out. 2019.

HOLMES, M. H.; MOW, V. C. The nonlinear characteristics of soft gels and hydrated connective tissues in ultrafiltration. Journal of Biomechanics, v. 23, n. 11, p. 1145–1156, 1 jan. 1990.

HORGAN, Cornelius O.; MURPHY, Jeremiah G. Limiting Chain Extensibility Constitutive Models of Valanis–Landel Type. Journal of Elasticity, v. 86, n. 2, p. 101–111, 1 fev. 2007.

HORGAN, Cornelius O.; SACCOMANDI, Giuseppe. Constitutive Models for Compressible Nonlinearly Elastic Materials with Limiting Chain Extensibility. Journal of Elasticity, v. 77, n. 2, p. 123–138, 1 nov. 2004.

HOSS, Leonardo; MARCZAK, Rogério J. A New Constitutive Model for Rubber-Like Materials. Mecánica Computacional, v. 29, n. 28, p. 2759–2773, 2010.

HUMPHREY, J. D.; YIN, F. C. P. On Constitutive Relations and Finite Deformations of Passive Cardiac Tissue: I. A Pseudostrain-Energy Function. Journal of Biomechanical Engineering, v. 109, n. 4, p. 298–304, 1 nov. 1987.

ISIHARA, Akira; HASHITSUME, Natsuki; TATIBANA, Masao. Statistical Theory of Rubber‐Like Elasticity. IV. (Two‐Dimensional Stretching). The Journal of Chemical Physics, v. 19, n. 12, p. 1508–1512, 1 dez. 1951.

JAMES, A. G.; GREEN, A.; SIMPSON, G. M. Strain energy functions of rubber. I. Characterization of gum vulcanizates. Journal of Applied Polymer Science, v. 19, n. 7, p. 2033–2058, 1975.

JAMES, Hubert M.; GUTH, Eugene. Theory of the Elastic Properties of Rubber. The Journal of Chemical Physics, v. 11, n. 10, p. 455–481, 1 out. 1943.

KALISKE, M.; HEINRICH, G. An Extended Tube-Model for Rubber Elasticity: Statistical-Mechanical Theory and Finite Element Implementation. Rubber Chemistry and Technology, v. 72, n. 4, p. 602–632, 1 set. 1999.

KHAJEHSAEID, H.; ARGHAVANI, J.; NAGHDABADI, R. A hyperelastic constitutive model for rubber-like materials. European Journal of Mechanics - A/Solids, v. 38, p. 144–151, 1 mar. 2013.

KHIÊM, Vu Ngoc; ITSKOV, Mikhail. Analytical network-averaging of the tube model:: Rubber elasticity. Journal of the Mechanics and Physics of Solids, v. 95, p. 254–269, 1 out. 2016.

KILIAN, H. G. A molecular interpretation of the parameters of the van der Waals equation of state for real networks. Polymer Bulletin, v. 3, n. 3, p. 151–158, 1980.

KILIAN, Hanns-Georg. Equation of state of real networks. Polymer, v. 22, n. 2, p. 209–217, 1 fev. 1981.

KNOWLES, J. K. The finite anti-plane shear field near the tip of a crack for a class of incompressible elastic solids. International Journal of Fracture, v. 13, n. 5, p. 611–639, 1977.

KORBA, Ahmed G.; BARKEY, Mark E. New Model for Hyper-Elastic Materials Behavior With an Application on Natural Rubber. In: ASME 2017 12TH INTERNATIONAL MANUFACTURING SCIENCE AND ENGINEERING CONFERENCE COLLOCATED WITH THE JSME/ASME 2017 6TH INTERNATIONAL CONFERENCE ON MATERIALS AND PROCESSING. Anais... American Society of Mechanical Engineers Digital Collection, 24 jul. 2017. Disponível em: <https://dx.doi.org/10.1115/MSEC2017-2792>. Acesso em: 13 jun. 2025

KUHN, Werner. Dependence of the average transversal on the longitudinal dimensions of statistical coils formed by chain molecules. Journal of Polymer Science, v. 1, n. 5, p. 380–388, 1946.

LAMBERT-DIANI, Julie; REY, Christian. New phenomenological behavior laws for rubbers and thermoplastic elastomers. European Journal of Mechanics - A/Solids, v. 18, n. 6, p. 1027–1043, 1 nov. 1999.

LIM, Goy Teck. Scratch behavior of polymers. Ph.D.—[S.l.: S.n.].

LION, Alexander. On the large deformation behaviour of reinforced rubber at different temperatures. Journal of the Mechanics and Physics of Solids, v. 45, n. 11, p. 1805–1834, 1 nov. 1997.

LOPEZ-PAMIES, Oscar. A new I1<math><msub is="true"><mi is="true">I</mi><mn is="true">1</mn></msub></math>-based hyperelastic model for rubber elastic materials. Comptes Rendus Mécanique, v. 338, n. 1, p. 3–11, 1 jan. 2010.

MANSOURI, M. R.; DARIJANI, H. Constitutive modeling of isotropic hyperelastic materials in an exponential framework using a self-contained approach. International Journal of Solids and Structures, v. 51, n. 25, p. 4316–4326, 1 dez. 2014.

MARTINS, J. A. C. et al. A numerical model of passive and active behavior of skeletal muscles. Computer Methods in Applied Mechanics and Engineering, Containing papers presented at the Symposium on Advances in Computational Mechanics. v. 151, n. 3, p. 419–433, 20 jan. 1998.

MEISSNER, Bohumil; MATĚJKA, Libor. A Langevin-elasticity-theory-based constitutive equation for rubberlike networks and its comparison with biaxial stress–strain data. Part I. Polymer, v. 44, n. 16, p. 4599–4610, 1 jul. 2003.

MEISSNER, Bohumil; MATĚJKA, Libor. A Langevin-elasticity-theory-based constitutive equation for rubberlike networks and its comparison with biaxial stress–strain data. Part II. Polymer, v. 45, n. 21, p. 7247–7260, 29 set. 2004.

MIEHE, C.; GÖKTEPE, S.; LULEI, F. A micro-macro approach to rubber-like materials—Part I: the non-affine micro-sphere model of rubber elasticity. Journal of the Mechanics and Physics of Solids, v. 52, n. 11, p. 2617–2660, 1 nov. 2004.

MIROSHNYCHENKO, Dmitri; GREEN, W. A. Heuristic search for a predictive strain-energy function in nonlinear elasticity. International Journal of Solids and Structures, v. 46, n. 2, p. 271–286, 15 jan. 2009.

MIROSHNYCHENKO, Dmitri; GREEN, W. A.; TURNER, D. M. Composite and filament models for the mechanical behaviour of elastomeric materials. Journal of the Mechanics and Physics of Solids, v. 53, n. 4, p. 748–770, 1 abr. 2005.

MOONEY, M. A Theory of Large Elastic Deformation. Journal of Applied Physics, v. 11, p. 582–592, 1 set. 1940.

NAROOEI, K.; ARMAN, M. Modification of exponential based hyperelastic strain energy to consider free stress initial configuration and Constitutive modeling. J. Comput. Appl. Mech., v. 49, n. 1, p. 189–196, 2018.

NUNES, L. C. S. Mechanical characterization of hyperelastic polydimethylsiloxane by simple shear test. Materials Science and Engineering: A, v. 528, n. 3, p. 1799–1804, 25 jan. 2011.

OGDEN, R. W. Large deformation isotropic elasticity – on the correlation of theory and experiment for incompressible rubberlike solids. Proc. R. Soc. Lond. A, v. 326, p. 565–584, 1972.

OTHMAN, Alias; GREGORY, M. J. A stress-strain relationship for filled rubber. 1 jan. 1990.

PEIXOTO, D. H. N. Estudo sobre diferentes medidas de deformação e apresentação da família seno-hiperbólica generalizada, com aplicação na modelagem de polímeros, biomateriais e tecidos biológicos moles. Belo Horizonte - MG: Universidade Federal de Minas Gerais, 14 jun. 2024.

PEIXOTO, Daniel Henrique Nunes; GRECO, Marcelo; VASCONCELLOS, Daniel Boy. A new family of strain tensors based on the hyperbolic sine function. Latin American Journal of Solids and Structures, v. 21, p. e529, 1 mar. 2024.

PENG, T. J.; LANDEL, R. F. Stored energy function of rubberlike materials derived from simple tensile data. Journal of Applied Physics, v. 43, 1 jul. 1972.

PLAGGE, J. et al. Efficient modeling of filled rubber assuming stress-induced microscopic restructurization. International Journal of Engineering Science, v. 151, p. 103291, 1 jun. 2020.

PUCCI, Edvige; SACCOMANDI, Giuseppe. A Note on the Gent Model for Rubber-Like Materials. Rubber Chemistry and Technology, v. 75, n. 5, p. 839–852, 1 nov. 2002.

RAGHAVAN, M. L.; VORP, David A. Toward a biomechanical tool to evaluate rupture potential of abdominal aortic aneurysm: identification of a finite strain constitutive model and evaluation of its applicability. Journal of Biomechanics, v. 33, n. 4, p. 475–482, 1 abr. 2000.

SETH, B. R. GENERALIZED STRAIN MEASURE WITH APPLICATIONS TO PHYSICAL PROBLEMS. Wisconsin Univ-Madison Mathematics Research Center: [S.n.]. Disponível em: <https://apps.dtic.mil/sti/citations/AD0266913>. Acesso em: 11 maio. 2023.

SHARIFF, M. H. B. M. Strain Energy Function for Filled and Unfilled Rubberlike Material. Rubber Chemistry and Technology, v. 73, n. 1, p. 1–18, 1 mar. 2000.

SINGH, Fateh; KATIYAR, V. K.; SINGH, B. P. A new strain energy function to characterize apple and potato tissues. Journal of Food Engineering, v. 118, n. 2, p. 178–187, 1 set. 2013.

SWANSON, S. R. A Constitutive Model for High Elongation Elastic Materials. Journal of Engineering Materials and Technology, v. 107, n. 2, p. 110–114, 1 abr. 1985.

TAKAMIZAWA, Keiichi; HAYASHI, Kozaburo. Strain energy density function and uniform strain hypothesis for arterial mechanics. Journal of Biomechanics, v. 20, n. 1, p. 7–17, 1 jan. 1987.

TANG, Dalin et al. 3D MRI-Based Anisotropic FSI Models With Cyclic Bending for Human Coronary Atherosclerotic Plaque Mechanical Analysis. Journal of Biomechanical Engineering, v. 131, n. 061010, 5 maio 2009.

TRELOAR, L. R. G. The elasticity of a network of long-chain molecules. I. Transactions of the Faraday Society, v. 39, p. 36–41, 1943a.

TRELOAR, L. R. G. The elasticity of a network of long-chain molecules—II. Transactions of the Faraday Society, v. 39, n. 0, p. 241–246, 1 jan. 1943b.

TRELOAR, L. R. G.; RIDING, G. A non-Gaussian theory for rubber in biaxial strain. I. Mechanical properties. Proceedings of the Royal Society of London. A. Mathematical and Physical Sciences, v. 369, n. 1737, p. 261–280, jan. 1997.

VALANIS, K. C.; LANDEL, R. F. The Strain‐Energy Function of a Hyperelastic Material in Terms of the Extension Ratios. Journal of Applied Physics, v. 38, n. 7, p. 2997–3002, 1 jun. 1967.

VAN DAM, Evelyne A. et al. Non-linear viscoelastic behavior of abdominal aortic aneurysm thrombus. Biomechanics and Modeling in Mechanobiology, v. 7, n. 2, p. 127–137, abr. 2008.

VERONDA, D. R.; WESTMANN, R. A. Mechanical characterization of skin—Finite deformations. Journal of Biomechanics, v. 3, n. 1, p. 111–124, 1 jan. 1970.

VITO, Raymond. A note on arterial elasticity. Journal of Biomechanics, v. 6, n. 5, p. 561–564, 1 set. 1973.

VOLOKH, K. Y.; VORP, D. A. A model of growth and rupture of abdominal aortic aneurysm. Journal of Biomechanics, v. 41, n. 5, p. 1015–1021, 1 jan. 2008.

WALL, Frederick T.; FLORY, Paul J. Statistical Thermodynamics of Rubber Elasticity. The Journal of Chemical Physics, v. 19, n. 12, p. 1435–1439, 1 dez. 1951.

WARNER, Harold R. Jr. Kinetic Theory and Rheology of Dilute Suspensions of Finitely Extendible Dumbbells. Industrial & Engineering Chemistry Fundamentals, v. 11, n. 3, p. 379–387, 1 ago. 1972.

WU, P. D.; VAN DER GIESSEN, E. On improved 3-D non-Gaussian network models for rubber elasticity. Mechanics Research Communications, v. 19, n. 5, p. 427–433, 1 set. 1992.

WU, P. D.; VAN DER GIESSEN, E. On improved network models for rubber elasticity and their applications to orientation hardening in glassy polymers. Journal of the Mechanics and Physics of Solids, v. 41, n. 3, p. 427–456, 1 mar. 1993.

XIANG, Yuhai et al. A general constitutive model of soft elastomers. Journal of the Mechanics and Physics of Solids, v. 117, p. 110–122, 1 ago. 2018.

YAMASHITA, Y.; KAWABATA, S. Approximated form of the strain energy-density function of carbon-black filled rubbers for industrial applications. J. Soc. Rubber Ind. Jpn., v. 65, n. 9, p. 517–528, 1992.

YEOH, O. H. Characterization of Elastic Properties of Carbon-Black-Filled Rubber Vulcanizates. Rubber Chemistry and Technology, v. 63, n. 5, p. 792–805, 1 nov. 1990.

YEOH, O. H. Some Forms of the Strain Energy Function for Rubber. Rubber Chemistry and Technology, v. 66, n. 5, p. 754–771, 1 nov. 1993.

YEOH, O. H.; FLEMING, P. D. A new attempt to reconcile the statistical and phenomenological theories of rubber elasticity. Journal of Polymer Science B Polymer Physics, v. 35, p. 1919–1931, 1 set. 1997.

ZHAO, Zihan; MU, Xihui; DU, Fengpo. Modeling and Verification of a New Hyperelastic Model for Rubber-Like Materials. Mathematical Problems in Engineering, v. 2019, n. 1, p. 2832059, 2019.